Product Specification - Ridgeback 2P backpacking tent

Document PS-RB2, version 1.2, 2 April 2026. Author: Lukas Brandt (Product Designer). Approver: Tom Albright (Product Director). Status: for sampling.

1. Product summary

A light two-person, two-door, semi-freestanding tent for backpackers and wild campers in UK conditions. It sits between the Fellwalker 3 (family trekking) and the discontinued Solo Tarn. Target launch 1 February 2027. Target RRP £279; target landed cost £61.50.

2. Key targets

Minimum trail weight (fly, inner, poles): 1.65 kg. Packed weight with pegs and bags: 1.89 kg. Packed size: 42 x 15 cm.

Floor: 220 x 130 cm (tapering to 110 cm at the foot). Peak height: 105 cm.

Two side doors, two porches of 0.8 m2 each.

Pitch time (one person, first-time user with instructions): under 4 minutes. Pitches fly-first or all-in-one.

Wind: stable in 50 mph gusts when fully guyed (test in the Avonmouth wind rig, then field test on Dartmoor).

3. Materials

Flysheet: 70D ripstop nylon, silicone outer / PU inner, hydrostatic head 3,000 mm minimum, fabric code NT-RS70-OL (Kestrel olive).

Inner: 40D no-see-um mesh upper (NT-MS40-GR) with 20D breathable nylon lower panels.

Groundsheet: 210T polyester PU 5,000 mm (NT-PL21-BK), bathtub height 12 cm.

Poles: 8.5 mm 7001-T6 aluminium, one hubbed main pole plus a short ridge pole. Shock cord replaceable.

Pegs: 10 x 6 g Y-section aluminium, plus 2 spare.

Seams: fully taped on fly and groundsheet.

4. Features

Two vents with rigid struts. Four inner pockets and one overhead lamp loop. Reflective guy lines (6 points). Colour-coded pole clips. Fly door hooks to hold doors open. Stuff sack with compression straps and printed pitching instructions.

5. Testing and standards

Hydrostatic head tested on every production batch (supplier certificate plus 1 in 20 in-house check). Flame resistance labelling where required for export markets. Seam strength and UV testing to our internal standard QS-04.

6. Timeline

Sample round 1: June 2026. Field testing with 12 staff and 6 ambassadors: July-August 2026. Sample round 2 (fixes): September 2026. Production order: November 2026. Warehouse arrival: early January 2027. Launch: 1 February 2027.

7. Open issues

1. Weight is 70 g over target in sample 0 - options: lighter pole hub or 15D mesh.

2. Olive colour matching between fly and door zips - agree a tolerance with the zip supplier.

3. Decide whether a footprint (£29 RRP) is sold separately or included.

4. Packaging: move to printed paper bag instead of polybag (aligns with the Tidewell carton change).

Change log

v1.0 initial specification. v1.1 floor width increased from 125 to 130 cm after staff feedback. v1.2 added wind test and packaging decision.